Steelhead –DRAFT

BDCP Logic Chains for Covered Fish Species

Note to Reviewers:

The following presents a draft set of straw BDCP biological objectives for steelhead.  Per the recommendations of the independent science review panel, the objectives have been structured to address specific stressors as identified in existing documents such as existing recovery plans, biological opinions, and/or DRERIP life history conceptual models.  A standardized table is used for each objective to provide specificity regarding the objective.  Terms used in the table such as “Indicator” and “Attribute” are defined in Attachment 1.  Additional components of the logic chain such as expected outcomes, conservation measures, and monitoring metrics are not presented herein.  However, portions of the objective table are specifically intended to provide information relevant for these additional components.  Efforts to link specific species objectives to broader natural community objectives and ecosystem objectives will be conducted once the species objectives have been reviewed and finalized.

Disclaimer:

- The Global Goals and Global Objectives presented below are not BDCP goals and objectives.  BDCP will contribute to the achievement of these global goals and objectives.

Table of Contents

Global Goal	2

Global Objectives	2

Stressors/Limiting Factors	3

Stressor #1: Habitat Loss and Modification	4

Stressor #2: Altered Flows	5

Stressor #3: Impingement and Entrainment	6

Stressor #4: Water Quality (Toxics, D.O., and Temperature)	7

Stressor #5: Predation	8

Stressor #6: Passage Impediments/Barriers	9

Stressor #7: Hatchery Effects	10

Stressor #8: Illegal Harvest	11

References	12

Attachment 1:   Objective Worksheet	13

# Global Goal

Removal of the Central Valley steelhead DPS from the Federal List of Endangered and Threatened Wildlife (NMFS 2009).  According to the NMFS draft recovery plan (2009), recovery and long-term sustainability requires:

- Adequate protection for replacement of losses due to natural mortality (disease and stochastic events);

- Sufficient genetic robustness to avoid inbreeding depression and allow for adaptation;

- Sufficient habitat (type, amount, and quality) for long-term population maintenance, and;

- Elimination or control of threats.

# Global Objectives

- Increase abundance;

- Increase spatial extent of key life stages;

- Restore life history/genetic diversity to historic/natural levels, and;

- Increase productivity (population growth rate = births-deaths).

# Stressors/Limiting Factors

The following stressors/limiting factors were adapted from Williams (2009) and the National Marine Fisheries Service (NMFS) Draft Recovery Plan for Chinook salmon and Steelhead (2009).  Not all of the stressors listed below are proposed to be addressed by BDCP.

| ID | Stressor | Summary Description |
| --- | --- | --- |
| Stressors Addressed by BDCP |  |  |
| 1 | Habitat loss and modification | Changes in the extent, access to, and or quality of habitat including habitat variability and food. |
| 2 | Altered flows | Modifications to Delta inflow and outflow rates and hydrodynamics resulting in deviations from historic migration pathways, delays and reduced survival. |
| 3 | Impingement and entrainment. | Impingement and entrainment at project and non-project facilities |
| 4 | Water quality (toxics, DO, temperature). | Water quality conditions affecting migration, growth rate, and reproductive success. |
| 5 | Predators/non-native invasive species. | Competition, predation, and alteration of habitat characteristics by non-native species. |
| 6 | Passage impediments | Barriers to migration (upstream and downstream). |
| 7 | Hatchery effects | Interbreeding or hatchery and wild-type species can reduce genetic fitness with long-term effects. |
| 8 | Illegal harvest | Direct mortality due to illegal harvest. |
| Stressors Not Addressed by BDCP |  |  |
| 9 | Access to historic spawning habitat. | Barriers to historic spawning habitat are predominately located outside of the BDCP planning area. In-delta migration and barriers addressed in Stressor #2, 4 and 6 above. |
| 10 | Climate Change. | Increases in ambient air temperatures resulting in increased water temperatures with negative effects on habitat suitability. |

### Stressor #1: Habitat Loss and Modification

Steelhead are thought to primarily use the Delta as migration habitat.  Acoustic tagging studies have shown that most steelhead smolts rapidly emigrate downstream and pass through the Delta in a matter of days. Use of the Delta for rearing may have been more important historically when habitat conditions were more suitable for such, but there is little to no data to support this conclusion, and there have been only very limited studies on current use of the Delta by steelhead.  Key characteristics of quality migration habitat for steelhead are: suitable downstream flows, complexity and cover for predator avoidance and resting, good water quality, and opportunistic feeding potential.

BDCP Objective #1

Increase extent, access to, availability and quality of migration habitat for juvenile steelhead.

| Relation to Global Objective | Increasing the extent, access to, availability and quality of migration habitat will improve juvenile survival in the Delta and growth (~increased survival of smolts in the Bay and nearshore ocean). |
| --- | --- |
| Indicator | Floodplain and channel margin riparian habitat Survivorship and outmigration |
| Locations | - Yolo Bypass, West Delta, Cosumnes/Mokelumne, South Delta - Sacramento River, Steamboat and Sutter sloughs - San Joaquin River (between Vernalis and Mossdale) |
| Attribute | - Extent, duration, and frequency, of access to activated floodplain habitat. - Extent and quality of riparian channel margin habitat - Survival of outmigrants |
| Quantity or State | AchieveYolo Bypass inundation frequency of 75 to 90% of years for a minimum of __ days that inundate __ acres (specify spatial extent) – see currently proposed action Increase spatial extent of tidal habitat (up to 65,000 acres). Create and/or enhance 20 miles of channel margin habitat. Increase presence of preferred prey. Need to explore spatial extent question - Further calibrate flow rate needed to get a significant fraction of the fish into Yolo bypass (telemetry study) |
| Time Frame | Floodplain Habitat: - within 10 years Channel Margin Habitat: - at least 5 miles by year 10 - at least 10 miles by year 20 - at least 15 miles by year 25 - at least 20 miles by year 30 |

### Stressor #2: Altered Flows

Modification of natural flow regimes has resulted in: changes in water temperatures; changes in fish community structures; and depleted flow necessary for migration, spawning, rearing, and flushing of sediments from spawning gravels (NMFS 2009).

Stability of flows and cold water releases by themselves do not dictate residency in O. mykiss. The most important factors are smolt to adult survival rate and asymptotic size. Stream survival and adult survival to repeat spawn are also important. (Satterthwaite et al. 2010).  A highly stable, productive stream with 100% of fish expressing anadromy is possible if smolt survival, asymptotic size and adult survival are high enough.

BDCP Objective #2

Provide hydrodynamic conditions that facilitate migration of juvenile steelhead, adults, and kelts

| Relation to Global Objectives | Improving outmigration success will: - Increase productivity (more return spawners); - Promote maintenance of life history/genetic diversity (by increasing the window of migration opportunity). Improved upstream migration will: - Increase productivity; - Promote maintenance of life history/genetic diversity (by increasing the window of migration opportunity). |
| --- | --- |
| Indicator | In Delta hydrodynamics Juvenile survival |
| Location | Sacramento and San Joaquin River |
| Attribute | River flows at Knights Landing, Freeport, Rio Vista and Vernalis. Net tidal flows (OMR flows, Delta outflow) Gate operations Outmigration success (eg. JPI) - Survival Knights Landing to Chips Island (using telemetry) and Vernalis to Chips Island. |
| Quantity or State | Positive trajectory in the survival indices of downstream migrating fish. Sustainable population level after recovery goal is achieved. |
| Time Frame | Within 10 years of permit issuance |

### Stressor #3: Impingement and Entrainment

According to NMFS (1997), entrainment of juvenile steelhead is one of the most ubiquitous causes of mortality in the Sacramento River and Delta. Unscreened water diversions and CVP and SWP pumping plants entrain juvenile steelhead, leading to fish mortality (NMFS 2009). Cooling water intakes at power plants in Antioch and Pittsburgh entrain and kill juvenile fish from mechanical and heat stress (CALFED 2000).

BDCP Objective #3

Reduce impingement and entrainment of juvenile steelhead.

| Relation to Global Objective | Reducing direct and indirect mortality associated with impingement, entrainment and salvage will have positive effects on: - Productivity - Life history/genetic diversity maintenance (restoration) |
| --- | --- |
| Indicator | Impingement and entrainment |
| Location | Power plants and water diversions within BDCP planning area. |
| Attribute | Impingement Entrainment Salvage |
| Quantity or State | Reduce impingement and entrainment by __% Don’t currently have data to estimate % entrainment for Steelhead. Could use hatchery Steelhead as a surrogate for estimating % entrainment (tagged fish) – technique used for other species. Or develop a better way to estimate adult numbers. Could also use tagging or telemetry study. |
| Time Frame | Within 10 years of permit issuance and maintained annually thereafter. |

### Stressor #4: Water Quality (Toxics, D.O., and Temperature)

The main potential toxicity components for steelhead are ammonia, pyrethroid pesticides, and copper (Williams 2009).  The effects of these contaminants include the suppression of immune competence, reduced growth and damage to the olfactory system (NMFS 1997, Williams 2009). High concentrations of ammonium can inhibit primary and secondary production with cascading trophic effects (NMFS 2009).

Water temperatures in the lower Sacramento and San Joaquin Rivers are generally too warm for steelhead rearing; these reaches are likely used only as migratory corridors (NMFS 2009).

Water quality in the San Joaquin River during periods of low flow is generally degraded due to high temperatures, heavy metals, and pesticides from drainage (NMFS 2009). Operation, usage and maintenance of the Stockton Deep Water Ship Channel (DWSC) create localized water quality impairments (dissolved oxygen, exposure to contaminants, noise, etc.) that may pose a threat to steelhead utilizing the San Joaquin River as a migratory corridor (NMFS 2009).   These DO and other water quality effects impair upstream migration and create barriers to migration (see Stressor #6).

BDCP Objective #4 – match winter-run

a.  Toxics - Reduce levels of ammonia, organophosphate, pyrethroid pesticides and copper in the Delta to levels below chronic and acute effect threshold for steelhead.

b. Water Temperature - Maintain water temperatures within the suitable range for juvenile and adult steelhead in upstream spawning areas.

| Relation to Global Objective | Improvements in key water quality parameters will positively effect: - Spatial extent of key life stages - Productivity - Life history/genetic diversity maintenance (restoration) |
| --- | --- |
| Indicator | Water quality parameters. |
| Location | Key migratory corridors, including upstream spawning and rearing areas of Sacramento River (for temperature) and the San Joaquin River |
| Attribute | - Concentration (µg/L) of; - ammonia, - pyrethroids, - copper, - organophosphates - Water temperature in the upper Sac river (°C) |
| Quantity or State Time Frame | TBD |

### Stressor #5: Predation

Predation is a threat to steelhead in the Delta where there are high densities of non-native predatory fish (NMFS 2009).  Modification of natural channel margins and riparian habitats, colonization of non-native SAV and FAV, as well as artificial instream structures may change the natural predator-prey dynamics favoring predators (NMFS 2009).

BDCP Objective #5

Reduce susceptibility to, and impact of predation by non-native predatory fish on juvenile outmigrants.

| Relation to Global Objective | Reducing predation of steelhead will increase survival of juveniles through the Delta |
| --- | --- |
| Indicator | Juvenile survival and predator abundance in a given area |
| Location | BDCP Planning area including Clifton Court Forebay and the mainstem of the Sacramento and San Joaquin River |
| Attribute | Survivorship Number of predators in a given area |
| Quantity or State | We have established an ambitious objective of reducing predation related mortality by 5%, recognizing our ability to detect and measure the effects of predation is limited, more research is needed, and that the value of reduced predation to salmonid populations is uncertain. |
| Time Frame | Within 10 years of permit issuance |

### Stressor #6: Passage Impediments/Barriers

Passage impediments block access to historic staging and spawning habitats and eliminate the spatial segregation of spawning habitat that historically existed for steelhead. These barriers exist primarily at low flows and likely impede upstream migration and potentially early migrating adult steelhead (Vanicek 1993).

Hydrodynamic conditions created by operation of the Delta Cross Channel (DCC) can delay or block juvenile salmon that outmigrate through the central Delta (NMFS 2009). Additionally, dissolved oxygen concentrations on the San Joaquin River near Stockton can be low enough to block migration of adults (Hallock et al. 1970; Alabaster 1989).  Usually this problem eases in late October.

BDCP Objective #6

Improve upstream and downstream passage for steelhead.

| Relation to Global Objective | Elimination of passage barriers will have positive effects on: - Productivity - Life history/genetic diversity maintenance (restoration) |
| --- | --- |
| Indicator | Upstream and downstream passage |
| Location | Yolo Bypass and Stockton Deep Water Ship Channel |
| Attribute | - Dissolved oxygen levels (mg/L) - Immigration rate and success - Emigration rate and success |
| Quantity or State | Increase immigration success by __%. Reduce migratory delays by ___% |
| Time Frame | Within 10 years of permit issuance. |

### Stressor #7: Hatchery Effects

There are two artificial propagation programs for steelhead in the Central Valley, which are contributing to increased abundance of the DPS (NMFS 2009). However, there is concern that the proportion of naturally produced fish is declining (NMFS 2009). Hatchery-origin and natural-origin steelhead have been managed as a single stock; mixing of hatchery and natural origin population components occurred through spawning at the hatchery and intermingling of natural spawners in Battle Creek (NMFS 2009).

Potential threats to natural steelhead posed by hatchery programs include: (1) mortality of

natural steelhead in fisheries targeting hatchery-origin steelhead; (2) competition for prey and habitat; (3) predation by hatchery-origin fish on younger natural fish; (4) genetic introgression by hatchery-origin fish that spawn naturally and interbreed with local natural populations; and (5) disease transmission (NMFS 2009).

BDCP Objective #7

Manage hatchery to minimize adverse genetic effects (eg. domestication) on natural producing steelhead.

| Relation to Global Objective | Reductions in genetic effects of hatcheries will primarily effect: - Life history/genetic diversity maintenance (restoration) |
| --- | --- |
| Indicator | Steelhead genetics |
| Location | BDCP Planning Area |
| Attribute | - Develop and implement a genetic management program to assess winter-run Chinook salmon population genetic variability. - Adjust and maintain management program as needed. - What wild genome are we protecting? |
| Quantity or State | TBD |
| Time Frame | Within ___ years of permit issuance. |

### Stressor #8: Illegal Harvest

BDCP Objective #8

Discourage illegal harvest of adult steelhead.

| Relation to Global Objective | Reductions in illegal harvest will reduce adult mortality and increase productivity. |
| --- | --- |
| Indicator | Illegal Harvest |
| Location | Upstream spawning areas |
| Attribute | Enforcement |
| Quantity or State | Increase number of patrol hours by 100% during migration and adult holding period (Year-round) |
| Time Frame | Within ___ years of permit issuance. |

# References

CALFED. (2000). Ecosystem Restoration Program Plan Final Programmatic EIS/EIR Technical Appendix.

National Marine Fisheries Service (NMFS). (1997). Proposed Recovery Plan for the Sacramento River Winter-Run Chinook Salmon. Long Beach, CA: National Marine Fisheries Service, Southwest Region.

NMFS. (1998). Factors Contributing to the Decline of Chinook Salmon: An Addendum to the 1996 West Coast Steelhead Factors for Decline Report. Portland, Oregon: Protected Resources Division, National Marine Fisheries Service.

NMFS. (2009). Public Draft Recovery Plan for the Evolutionary Significant Units of Sacramento River winter-run Chinook salmon and Central Valley spring-run Chinook salmon and the Distinct Population Segment of Central Valley steelhead. Sacramento Protected Resources Division. October 2009.

Satterthwaite, et. al. (2009). State-dependent Life History Models in a Changing (and regulated) Environment: Steelhead in the California Central Valley. Evolutionary Applications ISSN 1752-4571.

USACE and Reclamation Board. 1999. Sacramento and San Joaquin River Basins

Comprehensive Study Interim Report.

USDI, Reclamation, San Joaquin River Group Authority, USFWS, NMFS, and DWR. (1999). Meeting Flow Objectives for the San Joaquin River Agreement 1999-2010

Environmental Impact Statement and Environmental Impact Report.

Williams, J.G. (2009). Life-history conceptual model for Chinook salmon and steelhead (Oncorhynchus tshawytscha and O. mykiss) - Partial Review. 75 pp.  Sacramento-San Joaquin Delta Regional Ecosystem Restoration Implementation Plan. December 2009.

# Attachment 1:   Objective Worksheet

| Relation to Global Objective | How will the stressor-level objective contribute to achieving the global objective? |
| --- | --- |
| Indicator | What will be measured? Species, habitat, ecological process, physical condition… |
| Location | Where will it be achieved? |
| Attribute | What aspect of the indicator will be measured? Population size, density, cover, presence/absence, reproductive rate… |
| Quantity or State | What measurable condition or change is expected? Increase, decrease, maintain or limit negative impact? Quantity: 500 individuals, 20% cover, 30% increase … Quality: Weed-free, all life stages present, cover class 4… |
| Time Frame | When will this be achieved? |